Barry McIlroy
Maker of fine furniture

Care and maintenance of music stand

(i.) Stand is finished with Canauba wax and lemon oil. As the oil evaporates it leaves a hard layer of wax.
(ii.) The wax is resistant to finger marks. It is not resistant to liquids or dirt.

(iii.) Over time areas that are constantly touched may darken due substances on the skin. To remove this problem:

(i) Strip wax from all the piece of timber containing the soiled area with mineral turpentine – this may take a number of applications.
(ii) The piece should then be waxed liberally with a lint free cloth and allowed to dry.
(iii) Be careful not to build up too thick a layer of wax in corners as this will dry white (not clear) and be obvious – if this happens just remove with vigorous rubbing or judicious use of something sharp.
(iv) When dry buff vigorously with the felt cloth provided

(iv.) Occasionally an area will become a little dull – vigorous buffing should remedy this.
(v.) The surface of the stand may over a long period of time start to feel a little ‘rough’. To remedy this

(i) Gently rub down with 0000 steel wool.
(ii) 
Remove all dust with a cloth.
(iii) Apply a coat of wax and buff when dry.
NEVER USE ANY OTHER TYPE OF WAX OR WAX POLISH

23 Albany Crescent Surrey Hills 3127

9898 9119

